ANNEX

METHOD OF DETERMINATION OF CONSTRUCTION PRICE INDEX 
(Issued together with the Circular No.02/2011/TT-BXD dated 22/02/2011 of the Ministry Of Construction)

Construction price index is determined according to the following methods:

1. Determination of work construction price index

Price index of work construction is determined by total multiplication of the average proportion of construction costs, equipment, and other costs with the price index of corresponding construction part, the equipment part, and the other cost part of the selected representative works.

Price index of work construction (I) is calculated by the formula as follows:

I = PXDIXD + PTBITB+PCPKICPK	(1)

In which: PXD, PTB, PCPK - Average proportion of construction, equipment costs and other costs of the selected representative works;

Total of the above average proportion is by 1;

IXD, ITB, ICPK - Price index of parts of construction, the equipment, the other cost of selected representative works.

The method of determining the composition of the formula (1) as follows:

- Price index of parts of construction, the equipment, the other cost (IXD, ITB, ICPK) is determined according to guidance in item 2 below.

- Average proportion of construction, equipment costs, and other cost (PXD, PTB, PCPK) is determined as follows:

Average proportion of construction cost (PXD), equipment costs (PTB), other cost (PCPK) is determined by arithmetic average of the proportion of construction costs, the proportion of equipment costs, the corresponding proportion of other costs of the representative works in the type of work.

The proportion of construction costs, equipment costs, other costs of each representative work by the ratio between the costs of construction, equipment and other cost of the representative work compared to total of this cost of the work. The formula is determined as follows:

PXDi=GXDiGXDCTi		(2)

PTBi=GTBiGXDCTi		(3)

PCPKi=GCPKiGXDCTi	(4)

In which: PXDi, PTBi, PCPKi - The proportion of construction costs, equipment costs, other costs compared to this total cost of the ith representative work;

GXDi, GTBi, GCPKi - the costs of construction, equipment and other cost of the ith representative work;

GXDCTi - the total costs of construction, equipment and other cost of the ith representative work.

Data on construction costs, equipment costs and other expenses of the selected representative works are determined from collected statistics data.

2. Determination of index by cost structure

2.1. Price index of construction part (IXD) is determined by multiplication of price index of direct cost part (excluding other direct expenses) multiplied by the coefficient related to the remaining cost items on the cost component of materials, manpower and construction machines in construction cost.

IXD = ITT H	(5)

In which:

ITT - Price index of direct cost part (excluding other direct expenses) in construction cost of the representative works;

H – Coefficient of the remaining cost items in the construction costs including other direct cost, general cost, advance calculated taxable cost, value added tax calculated on cost of materials, manpower and construction machine in the cost of construction of the representative works.

Price index of the direct cost part (excluding other direct costs) (ITT) is determined by total multiplication of the average proportion of the cost of building materials, manpower, construction machines in the direct cost with the price index of materials, manpower, construction machine respectively is determined by the following formula:

ITT = PVL xKVL + PNC xKNC + PMTC xKMTC	(6)

In which: PVL, PNC, PMTC - the average proportion of the cost of materials, manpower, construction machines in the direct cost of the representative works;

Total average proportion mentioned above is by 1.

K VL , K NC , K MTC - Price index of work building materials, manpower, construction machine in the direct cost of the representative works.

Method of determining the composition of the formula (6) as follows:

- Price index of work building materials, manpower, construction machine (K VL , K NC , K MTC ) is determined according to guidance in item 3 below.

- The average proportion of the cost of materials, manpower, construction machines in the direct cost (PVL, PNC, PMTC) is determined as follows:

The average proportion of the cost of materials (PVL), manpower (PNC), and construction machines (PMTC) is determined by average of the proportion of cost of materials, manpower, construction machines of the selected representative works.

Proportion of costs of materials, manpower, construction machines of each representative work by the ratio between the costs of materials, manpower, and construction machines compared to total of these costs of the representative work. The formula is determined as follows:

|  |  | (7) |
| --- | --- | --- |
|  |  | (8) |
|  |  | (9) |

In which:

PVLi, PNCi, PMTCi - Proportion of costs of materials, manpower, and construction machines of the ith representative work;

GVLi, GNCi, GMTCi - Costs of materials, manpower, and construction machines in the direct cost of the ith representative work;

GTTi - The total costs of materials, manpower, and construction machines of the ith representative work.

Costs of materials, manpower, and construction machines are determined based on the volume of performed construction work, the norms and unit price of work construction estimates, quotation, materials quotation, manpower price announcement, machine price and construction equipment.

Coefficient related to the remaining cost items (H) in construction costs is determined by the ratio of the total multiplication of the item coefficient calculated on materials, manpower and construction machines multiplied by the proportion of corresponding costs at the time of comparison and total multiplication of the coefficient with their cost proportion at the original time.

Coefficient H may be determined as follows:

|  |  | (10) |
| --- | --- | --- |

In which:

- The remaining cost items coefficient (other direct costs, general cost, advance calculated taxable costs, value added tax) in construction costs is calculated on costs VL, NC, MTC at the time of comparison;

- The remaining cost items coefficient (other direct costs, general cost, advance calculated taxable costs, value added tax) in construction costs is calculated on costs VL, NC, MTC at the original time;

- Proportion of costs of materials, manpower, and construction machines at the time of comparison.

The proportion of the cost of each type of cost of material, manpower and construction machine in the direct cost at the time of comparison is determined by the proportion corresponding cost at the original time multiplied by its price index, divided by price index of the direct cost part.

|  |  | (11) |
| --- | --- | --- |
|  |  | (12) |
|  |  | (13) |

Coefficient related to the remaining cost items in construction costs includes: other direct costs, general costs, the cost of advance calculated taxable income, value added tax calculated on the costs of materials, manpower the construction machines determined based on guidance of the estimation of construction costs issued at the original time and the time of comparison and the type of work.

2.2. Price index of work equipment part (ITB) is determined by total multiplication of the average proportion of the costs of major equipment procurement, costs of installation and test, and correction of the major equipment (if any) multiplied by the coefficient of variation of corresponding costs said above of the selected representative works.

(14)

In which:

PSTB, PLĐ - The average proportion of the costs of major equipment procurement, costs of installation and test, and correction of the major equipment (if any) of the selected representative works;

KSTB, KLĐ - Coefficient of variation of the costs of major equipment procurement, costs of installation and test, and correction of the major equipment (if any) of the selected representative works.

Coefficient of variation of the cost of major equipment procurement is determined by ratio between costs of average equipment procurement at the time of comparison with the original time.

Price of equipment is determined by the method of investigation, statistics of price data of major equipment with bulk amount, high prices and much volatility on the market, or can be determined on the basis of reference of inflation rate of equipment, or calculated by the inflation factor of the equipment manufacturing structure.

The major equipment is the equipment with a large proportion of cost in the cost of equipment procurement. For example, for civil construction works: elevator, air conditioning systems etc, for the industrial construction works: major technological chain of production v.v.

Coefficient of variation of the costs of installing equipment, test, and correction of equipment (if any) is determined as for the construction price index. The proportion of the cost of equipment installation, test, and correction of equipment (if any) can be calculated by 6-10% of the cost of equipment procurement.

2.3. Price index of other cost part (ICPK) is determined by total multiplication of the average proportion of the major cost items in other costs of representative works multiplied by the coefficient of variation of corresponding cost items; it is determined by the following formula:

|  |  | (15) |
| --- | --- | --- |

In which:

PKMKs - The average proportion of the sth major cost items in total costs of major items of other cost part of representative works;

KKMKs - The coefficient of cost variation of the sth major cost items in total costs of representative works;

e – The number of major cost items of other cost part of representative works.

The major cost items in the other cost of the representative works represented are the cost items obtaining a large proportion in other costs of the representative works, for example, for civil construction works, the major cost items in other costs such as cost of construction survey, construction design cost, the cost of making construction investment project, project management cost,…

For the cost items obtaining a proportion of less than 1,5% in total other costs of the representative works may be not required to review.

For some other cost items based on the cost of construction or equipment cost, their coefficient of variation is taken by the price index of construction part or price index of the corresponding equipment part.

For some other cost items based on the total cost of construction or equipment cost, their coefficient of variation is taken by the average of price index of construction part or price index of the equipment part.

3. Determination of price index by cost factors

3.1. Price index of work building materials (K VL) is determined by total multiplication of the cost proportion of each type of major material multiplied by price index of the corresponding type of major material. Price index of work building materials at the time of comparison is as follows:

|  |  | (16) |
| --- | --- | --- |

In which:

Pvlj - The average cost proportion of each type of the jth major building material in total costs of types of major building materials of the representative works;

K VLj – Price index of type of the jth major building material;

m – Number of types of major building materials.

The average cost proportion (Pvlj) of type of the jth major building material in total costs of types of major building materials by average of cost proportion of types of the jth major building material of the representative works.

Total cost proportion of type of the major building material is by 1.

The cost proportion of each type of the jth major building material of each representative work is calculated by ratio between costs of type of the jth major building material compared to total costs of type of the major building materials in the direct cost of the representative work, it is determined as follows:

|  |  | (17) |
| --- | --- | --- |

In which:

- Cost proportion of type of the jth major building material of the representative work i;

- Cost of type of the jth major building material of the representative work i.

The major building materials prescribed include: cement, building sand, stone, construction wood, brick, tiles, construction steel, covering materials, electricity materials, water materials, asphalt, and complete materials.

Depending on the specific characteristics and nature of each construction work, type of major building material can be added to calculate appropriately.

Price index of types of building materials (K VLj) is calculated by average of the price index of types of building materials included in the group of the materials.

Price index of each type of material in the group is determined by the ratio between the average prices to the field of the type of building material at the time of comparison compared with the original time.

3.2. Price index of work construction manpower (K NC) is determined by average of price indexes of construction manpower of the major workmen of the work or type of work.

The type of major construction manpower is defined as follows: unskilled workers, mason workers, manpower for concrete work, carpentry manpower, manufacturing manpower, manpower of manufacture, erection of steel, and manpower of electrical installation, installation of water, road construction workers.

Depending on the specific characteristics and nature of each work/type of construction work select the type of major manpower for calculating the price index of construction manpower accordingly.

Price index of type of construction manpower is determined by the ratio between daily wages of average workman level at the time of comparison with the original time.

3.3. Price index of work construction machine (K MTC) is determined by total multiplication of the average proportion of the cost of group of major construction machines multiplied by price index of the group of major work construction machines, specifically as follows:

|  |  | (18) |
| --- | --- | --- |

In which:

PMk - The average proportion of the cost of group of kth major construction machines in total costs of the group of major construction machines of the representative works;

KMk – The price index of the group of kth major construction machines of the representative works;

f – Number of the group of major construction machines of the representative works.

The average proportion of the cost of group of kth major construction machines in total costs of the group of major construction machines (PMk) by average proportion of the cost of group of kth major construction machines of the representative works.

Total proportion of the cost of group of major construction machines is by 1.

Proportion of the cost of each group of major construction machines of each representative work is calculated by ratio between the costs of the group of major construction machines compared to total cost of groups of major construction machines in direct costs of the representative works:

|  |  | (19) |
| --- | --- | --- |

In which:

- The proportion of the cost of group of kth major construction machines of the ith representative work;

- The cost of group of kth major construction machines of the ith representative work.

The group of major construction machines as prescribed includes: group of soil-related machines, group of transport machines, group of lifting machines, group of concrete-related machines, group of metal-processing machines, group of pile-related machines, group of digging-tunnel machines, group of making-road machines.

Depending on the specific characteristics and nature of each construction work, the group of major construction machines can be added to calculate appropriately.

Price index of group of construction machines (K Mk) is calculated by average of price indexes of machine shift and construction equipment of types of machines and construction equipment included in the group.

Price indexes of machine shift and construction equipment of each type of machine and construction equipment is determined by ratio between machine shift price and construction equipment of such type of machine and construction equipment at the time of comparison compared with the original time.

4. Synthesis of determination of price index for localities (in case of having division of calculation area)

In the case the local has division of several areas to determine the construction price index, the synthesis of the price index of local’s areas into the local price index, and it is implemented by the following formula:

|  |  | (20) |
| --- | --- | --- |

In which:

PKVt – Proportion of building investment capital value of the local t in total construction investment capital value of the local in the year preceding the year of calculation;

IKVt – Construction price index of the area t;

n – Number of areas of the local.

The calculation of construction price index of area is implemented as mentioned above.

5. The example on calculation of construction price index

For example, below are the results of calculations determining the construction price index at a local. The origin time selected is in 2006, the time of comparison is by quarter.

After making a list of works belonging to the type of works required to be calculated; selection of works representing each type of work; data and figure collection at the original time and time of calculation, determination of the construction price index.

The following are the examples on the calculation for determining the construction price index for type of housing construction:

The calculation of work construction price index is implemented by the following steps:

Step 1: Conversion of the cost data collected at the time of the original choice for the determination of the proportion of costs.

- Data collection of total investment rate, cost estimates of a number of representative works of the type of housing project at the selected areas. After carrying out the conversion of costs to the plan of cost price at the selected time as origin, we have investment capital value and detail of cost items of representative works.

For example, detail of cost items constitutes construction investment capital of the representative work number one as follows:

+ With work construction investment capital (GXDCT1) is: 53,093,482,000 dong

In which:

- Construction cost (GXD1) :	43,235,591,000 dong

- Equipment cost (GTB1):	3,746,281,000 dong

+ Cost for equipment procurement: 	3,466,036,000 dong

+ Cost for equipment installation and testing, correction: 	280,245,000 dong

- Other costs (GCPK1):	6,111,610,000 dong

- Cost of materials, manpower and construction machines in direct costs and costs of each type of material, manpower, major construction machine group as presented in Table 1.

Step 2: Determination of cost proportion

a. Determination of the cost proportion by cost structure and cost factors for each representative work.

* Calculation of the cost structures of construction, equipment and other cost items of representative work number 1 (by formula 2, 3, 4), the results as follows:

- The proportion of construction cost (PXD1):	81,43%

- The proportion of equipment cost (PTB1):	7,06%, in which:

+ The proportion of equipment procurement cost:	92,52%

+ The proportion of cost of equipment installation, testing, calibration: 7,48%

- The proportion of other cost (PCPK1):	11,51%, in which:

+ The proportion of cost of construction survey: 	17%

+ The proportion of cost of work design: 	18%

+ The proportion of cost of project management:	65%

The proportion of the cost structure of other representative works calculated similar to the method of calculation for the representative housing project number one as mentioned above.

* Determination of the cost structure of materials, manpower, construction machines in direct cost; cost structure of type of material, major construction machine group of the work 1 (Table 1);

Table 1. Calculation of cost proportion of major materials, major group of construction machines of the work number 1 according to the plan of cost price in 2006

| No. | Names of materials, manpower, group of construction machines | Value (dong) | cost proportion of materials, manpower, group of construction machine | cost proportion of type of materials, manpower, major group of construction machine |
| --- | --- | --- | --- | --- |
| I | Type of materials | 21.172.067.758 | 61,75% | 100,00% |
| 1 | Wood | 2.629.570.816 |  | 12,42% |
| 2 | Building sand | 302.760.569 |  | 1,43% |
| 3 | Construction steel | 7.501.263.607 |  | 35,43% |
| 4 | Tiles | 472.137.111 |  | 2,23% |
| 5 | Brick | 1.084.009.869 |  | 5,12% |
| 6 | Cement | 2.646.508.470 |  | 12,50% |
| 7 | Building stone | 774.897.680 |  | 3,66% |
| 8 | Electricity materials | 2.650.742.883 |  | 12,52% |
| 9 | Water materials | 2.026.166.884 |  | 9,57% |
| 10 | Architecture materials | 946.391.429 |  | 4,47% |
| 11 | Covering materials | 137.618.440 |  | 0,65% |
| II | Type of manpower | 5.760.173.900 | 16,80% | 100,00% |
| III | Group of construction machines | 7.354.507.747 | 21,45% | 100,00% |
| 1 | Group of lifting machines | 2.443.902.924 |  | 33,23% |
| 2 | Group of concrete-related machines | 2.594.670.333 |  | 35,28% |
| 3 | Group of metal processing machines | 838.413.883 |  | 11,40% |
| 4 | Group of soil-related machines | 286.090.351 |  | 3,89% |
| 5 | Group of transport machines | 218.428.880 |  | 2,97% |
| 6 | Group of pile-related machines | 973.001.375 |  | 13,23% |
|  | Total value of materials, manpower, machines for construction | 34.286.749.405 |  |  |

The cost structure of the other representative works in the type of work is calculated the same method of calculation for the representative housing construction number 1.

b. Determination of the proportion by the cost structure and the average cost factors for this type of work.

After an average of the proportion of the representative works to be of type of housing work, it has the proportion of average costs as follows:

- The proportion of construction cost (PXD):	83,43%

- The proportion of equipment cost (PTB):	8,03%, in which:

+ The proportion of equipment procurement cost:	94,00%

+ The proportion of cost of equipment installation, testing, correction: 6,00%

- The proportion of other cost (PCPK):	8,54%, in which:

+ The proportion of cost of construction survey: 	15%

+ The proportion of cost of work design: 	29%

+ The proportion of cost of project management:	56%

- The proportion of the average costs of building materials, construction manpower, construction machine and proportion of the average cost of type of major building materials, group of major construction machines (Table 2).

Table 2. The proportion of the average costs of building materials, construction manpower, construction machines and type of major building materials, group of major construction machines according to the plan of cost price in 2006

| No. | Names of materials, manpower, construction machines | Cost proportion of materials, manpower, construction machines | Cost proportion of major materials, group of major construction machines |
| --- | --- | --- | --- |
| I | Type of materials | 64,33% | 100,00% |
| 1 | Wood |  | 4,90% |
| 2 | Building sand |  | 1,63% |
| 3 | Construction steel |  | 42,17% |
| 4 | Tiles |  | 5,33% |
| 5 | Brick |  | 4,63% |
| 6 | Cement |  | 10,78% |
| 7 | Building stone |  | 2,36% |
| 8 | Electricity materials |  | 8,39% |
| 9 10 11 | Water materials Architecture materials Covering materials |  | 8,30% 10,82% 0,69% |
| II | Type of manpower | 24,12% | 100,00% |
| III 1 2 3 4 5 6 | Group of construction machines Group of lifting machines Group of concrete-related machines Group of metal processing machines Group of soil-related machines Group of transport machines Group of pile-related machines | 11,55% | 100,00% 40,15% 27,44% 16,75% 4,88% 3,98% 6,80% |

Step 3: Determination of work construction price index.

a. Determination of price index of type of major building materials, manpower, construction machines.

Table 3 and Table 4 are the examples of calculating method of price index for types of materials as building sand and group of concrete-related machines.

Table 3. Calculation of price index of building sand

| No. | Type of materials | Unit | Original year | Quarter I/2010 |  | Quarter II/2010 |  | Quarter III/2010 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Price (dong) | Price (dong) | Index (%) | Price (dong) | Index (%) | Price (dong) | Index (%) |
| 1 | 2 | 3 | 4 | 5 | 6 = 5/4 x 100% | 7 | 8 = 7/4 x 100% | 9 | 10 = 9/4 x 100% |
| 1 | Yellow sand | m3 | 80.000 | 120.000 | 150,00 | 122.000 | 152,50 | 130.000 | 162,50 |
| 2 | Building, plastering sand | m3 | 65.000 | 95.000 | 146,15 | 91.000 | 140,00 | 94.000 | 144,62 |
| 3 | Scraping sand | m3 | 31.000 | 40.000 | 129,03 | 39.000 | 125,81 | 42.000 | 135,48 |
|  | Building sand index, % |  |  | 141,73 |  | 139,44 |  | 147,53 |  |

Note: Materials price at the time of comparison is the average price by quarter

Similar calculation to determine price index for other types of materials

Table 4. Calculation of price index for Group of concrete-related machines

| No. | Type of machines | Unit | Original year | Quarter I/2010 |  | Quarter II/2010 |  | Quarter III/2010 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Price (dong) | Price (dong) | Index (%) | Price (dong) | Index (%) | Price (dong) | Index (%) |
| 1 | 2 | 3 | 4 | 5 | 6 = 5/4 x 100% | 7 | 8 = 7/4 x 100% | 9 | 10 = 9/4 x 100% |
| 1 | Concrete mixing machine 250litre | shift | 91.325 | 157.420 | 172,37 | 157.420 | 172,37 | 157.420 | 172,37 |
| 2 3 4 5 | Automobiles for transport to mix 10,7m3 vibratory plate compactor 1KW vibrator cylinder 1,5KW Self-operating pump BT 50m3/h | shift shift shift shift | 1.753.712 60.268 62.997 1.994.830 | 2.610.806 113.225 115.885 2.805.878 | 148,87 187,87 183,95 140,66 | 2.610.806 113.225 115.885 2.805.878 | 148,87 187,87 183,95 140,66 | 2.610.806 113.225 115.885 2.805.878 | 148,87 187,87 183,95 140,66 |
|  | Price index of Group of concrete-related machines (%) |  |  | 166,75 |  | 166,75 |  | 166,75 |  |

Note: Price of construction machine shift in this example is determined according to guidance of the Ministry of Construction

Similar calculation to determine price index for other groups of construction machines

b. Determination of price index by cost factors

The determination of the price index for work building materials, construction machine is done by the formula (16, 18). Table 5 is an example of calculating the price index of work building materials, construction manpower, and construction machines.

Table 5. Calculation of the price index of work building materials, construction manpower, and construction machines

| No. | Contents | Proportion | Price Index (%) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Quarter I/2010 | Quarter II/2010 | Quarter III/2010 |
| I 1 2 3 4 5 6 7 8 9 10 11 | Types of materials Wood Building sand Construction steel Tiles Brick Cement Building stone Electricity materials Water materials Architecture materials Covering materials | 4,90% 1,63% 42,17% 5,33% 4,63% 10,78% 2,36% 8,39% 8,30% 10,82% 0,69% | 132,86 141,73 159,46 139,39 188,50 137,06 129,31 126,68 126,34 133,67 115,87 | 132,86 139,44 168,62 140,37 189,50 140,35 132,54 131,33 126,34 137,88 118,77 | 132,86 147,53 169,05 140,26 191,32 142,43 133,75 139,35 127,63 138,89 119,45 |
|  | Work building material price index |  | 146,43 | 151,65 | 153,18 |
| II 1 2 3 4 | Types of manpower Mason manpower Carpentering manpower Steel erecting manpower Concrete manpower |  | 234,12 234,12 234,12 234,12 | 234,12 234,12 234,12 234,12 | 234,12 234,12 234,12 234,12 |
|  | Work building manpower price index (1+...+4)/4 |  | 234,12 | 234,12 | 234,12 |
| III 1 2 3 | Group of construction machines Group of lifting machines Group of concrete-related machines Group of metal processing machines | 40,15% 27,44% 16,75% | 138,67 166,75 158,65 | 138,67 166,75 158,65 | 138,67 166,75 158,65 |
| 4 5 6 | Group of soil-related machines Group of transport machines Group of pile-related machines | 4,88% 3,98% 6,80% | 132,17 141,80 149,54 | 132,17 141,80 149,54 | 132,17 141,80 149,54 |
|  | Work construction machines price index |  | 150,27 | 150,27 | 150,27 |

Note: Price index of type of construction manpower is determined on the basis of comparison with the area average minimum wage stipulated by the State at the time of comparison and the original time.

K VL _ I / 2010 = 132,86 x 4,90% + 141,73 x 1,63% + ... + 115,87 x 0,69% = 146,43%

K MTC _ I / 2010 = 138,67 x 40,15% + 166,75 x 27,44% + ...+  149,54 x 6,80% = 150,27%

c. Determination of price index by cost structure

- The determination of the price index of the direct cost part (excluding other direct costs) under the formula number (6). Calculation results of price index of the direct cost part (excluding other direct costs) are presented in Table 6 below.

Table 6. Calculation of direct cost price index (excluding other direct costs)

| No. | Contents | Proportion | Price Index (%) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Quarter I/2010 | Quarter II/2010 | Quarter III/2010 |
| 1 2 3 | Work building materials Work building manpower Work construction machines | 64,33% 24,12% 11,55% | 146,43 234,12 150,27 | 151,65 234,12 150,27 | 153,18 234,12 150,27 |
|  | Price index of direct cost part |  | 168,02 | 171,38 | 172,37 |

I TT _ I / 2010 = 146,43 x 64,33% + 234,12 x 24,12% + 150,27 x 11,55% = 168,02%

- Determination of coefficient H: coefficient (H) relating to the remaining cost items on the costs of materials, manpower and construction machines is calculated by formula (10). Results of calculations are presented in Table 7.

Table 7. The calculation of coefficient (H) relating to the remaining cost items on the costs of materials, manpower and construction machines

| No. | contents | Original proportion | Item’s coefficient at original time | Quarter I/2010 |  |  | Quarter II/2010 |  |  | Quarter III/2010 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Price index (%) | Proportion at QI/2010 | Item’s coefficient | Price index (%) | Proportion at QII/2010 | Item’s coefficient | Chỉ số giá (%) | Proportion at QIII/2010 | Item’s coefficient |
| 1 | Materials | 0,64 | 1,26 | 146,43 | 0,56 | 1,27 | 151,65 | 0,57 | 1,27 | 153,18 | 0,57 | 1,27 |
| 2 | Manpower | 0,24 | 1,26 | 234,12 | 0,34 | 1,27 | 234,12 | 0,33 | 1,27 | 234,12 | 0,33 | 1,27 |
| 3 | Construction machine | 0,12 | 1,26 | 150,27 | 0,10 | 1,27 | 150,27 | 0,10 | 1,27 | 150,27 | 0,10 | 1,27 |
|  | Coefficient H |  |  | 1,01 |  |  | 1,01 |  |  | 1,01 |  |  |

Note: The coefficient of remaining cost items in the construction cost calculated on costs of materials, manpower, construction machines at the original time and  comparision time determined on the basis of regulations on the establishment and management of construction investment costs. For example, the results of determination are presented in Table 8 and Table 9.

Table 8. The total remaining cost items calculated on the cost of materials, manpower, and construction machines for civil projects (determined at the chosen time as origin)

| No. | Cost | Materials | Manpower | Construction machine |
| --- | --- | --- | --- | --- |
| I 1 2 3 4 II III IV V | DIRECT COST Material cost Manpower cost Construction machine cost Other direct cost Subtotal direct cost T=VL+NC+M GENERAL COST CPC=6,0%*T ADVANCE CALCULATED PAYABLE INCOME TL=5,5%*(T+CPC) Before-tax construction estimated value Z=T+CPC+TL OUTPUT VAT (Z*10%) After-tax estimated value (Z+VAT) COST FOR TEMPORARY SHELTER AT THE FIELD FOR ACCOMMODATION AND CONSTRUCTION ADMINISTRATION TOTAL | 1,000 0,015 1,015 0,061 0,059 1,135 0,114 1,249 0,012 1,261 | 1,000 0,015 1,015 0,061 0,059 1,135 0,114 1,249 0,012 1,261 | 1,000 0,015 1,015 0,061 0,059 1,135 0,114 1,249 0,012 1,261 |

Table 9. The total remaining cost items calculated on the cost of materials, labor and construction machines for construction, civil engineering (Determined at the comparison time)

| No. | Cost | Materials | Manpower | Construction machine |
| --- | --- | --- | --- | --- |
| I 1 2 3 4 II | DIRECT COST Material cost Manpower cost Construction machine cost Other direct cost Subtotal direct cost T=VL+NC+M GENERAL COST CPC=6,5%*T | 1,00 0,02 1,02 0,07 | 1,00 0,02 1,02 0,07 | 1,00 0,02 1,02 0,07 |
| III IV V | ADVANCE CALCULATED PAYABLE INCOME TL=5,5%*(T+CPC) Before-tax construction estimated value Z=T+CPC+TL OUTPUT VAT (Z*10%) Estimated value after tax (Z+VAT) COST FOR TEMPORARY SHELTER AT THE FIELD FOR ACCOMMODATION AND CONSTRUCTION ADMINISTRATION TOTAL | 0,06 1,15 0,11 1,26 0,01 1,27 | 0,06 1,15 0,11 1,26 0,01 1,27 | 0,06 1,15 0,11 1,26 0,01 1,27 |

- Price index of construction part is determined by formula number (5). Calculation results are presented in Table 10

Table 10. Calculating the price index of construction part (%)

| No. | Contents | Quarter I/2010 | Quarter II/2010 | Quarter III/2010 |
| --- | --- | --- | --- | --- |
| 1 2 | Price index of direct cost (excluding other direct costs) (%) Coefficient relating to remaining item cost H | 168,02 1,01 | 171,38 1,01 | 172,37 1,01 |
|  | Price index of Construction part | 169,65 | 173,04 | 174,04 |

- Price index of equipment part is determined by formula number (14). Calculation results are presented in Table 11

Table 11. Calculating the price index of equipment part (%)

| No. | Content | Coefficient | Coefficient of Cost Variation (% |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Quarter II/2010 | Quarter II/2010 | Quarter II/2010 |
| 1 | Purchasing equipment | 94% | 121,23 | 121,23 | 121,23 |
| 2 | Installing, testing and calibrating | 6% | 155,65 | 160,04 | 160,04 |
|  | Price index of equipment part |  | 123,30 | 123,56 | 123,56 |

- Price index of other costs is determined by formula number (15). Calculation results are presented in Table 12

Table 12. Calculating the price index of other costs

| No. | Content | Coefficient | Coeficient of Cost Variation (% |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Quarter I/2010 | Quarter II/2010 | Quarter III/2010 |
| 1 2 3 | Construction Survey Construction Design Project Management Cost | 15% 29% 56% | 234,12 169,65 151,44 | 234,12 173,04 154,29 | 234,12 174,04 155,12 |
|  | Price index of other costs |  | 169,12 | 171,70 | 172,46 |

c. Determining the price index of construction work

Price index of construction work is determined by formula number (1). Calculation results are presented in Table 13

Table13. Calculating the price index of construction work

| No | Contents | Coefficient | Price index (%) |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Quarter I/2010 | Quarter II/2010 | Quarter III/2010 |
| 1 | Construction | 83,43% | 169,65 | 173,04 | 174,04 |
| 2 | Equipments | 8,03% | 123,30 | 123,56 | 123,56 |
| 3 | Other costs | 8,54% | 169,12 | 171,70 | 172,46 |
|  | Price index of construction work |  | 165,88 | 168,95 | 169,85 |

I I / 2010 = 169,65 x 83,43% + 123,30 x 8,03% + 169,12 x 8,54% = 165,88%

Above is an example of calculating construction price index with comparison time upon quarter. In case price index with the comparison time according to month or year need to be determined, the calculations similar to as mentioned above is applied but the price of inputs (cost of materials, labor, and construction machine shifts) is the average price by month or by year respectively.

Form of synthesizing results for publication:

A. Price index of construction project

(Year 2006=100)

Calculated unit: %

| No. | Type of project | Quarter I/2010 | Quarter II/2010 | Quarter III/2010 |
| --- | --- | --- | --- | --- |
| I | Civil Construction project | 165,878 | 168,949 | 169,847 |

B. Price index by cost structure

(Year 2006=100)

Calculated unit: %

| No. | Type of project | … | Quarter II/2010 |  |  | Quarter III/2010 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Price index of building part | Price index of equipment part | Price index of other costs | Price index of building part | Price index of equipment part | Price index of other costs |
| I | Civil Construction project |  |  |  |  |  |  |  |
| 1 | Housing project |  | 173,04 | 123,56 | 171,70 | 174,04 | 123,56 | 172,46 |

C. Price index by cost factor

(Year 2006=100)

Calculated Unit:%

| No. | Type of project | … | Month 2/2010 |  |  | Month 3/2010 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Price index of building material | Price index of manpower | Price index of construction machines | Price index of building material | Price index of manpower | Price index of construction machines |
| I | Civil Construction project |  |  |  |  |  |  |  |
| 1 | Housing project |  | 145,65 | 234,12 | 150,27 | 146,35 | 234,12 | 150,27 |

D. Price index of some material, manpower, group of construction machines

(Year 2006=100)

Calculated unit: %

| No. | Type of material, manpower, construction machines | Month 1/2010 | Month 2/2010 | Month 3/2010 |
| --- | --- | --- | --- | --- |
| I | Type of material |  |  |  |
| 1 | Wood | 131,36 | 131,36 | 131,36 |
| 2 | Building sand | 140,63 | 140,63 | 141,52 |
| 3 | Construction steel | 157,48 | 158,61 | 159,15 |
| 4 | Tiles | 132,39 | 132,39 | 133,26 |
| 5 | Brick | 188,50 | 189,50 | 189,52 |
| 6 | Cement | 137,06 | 137,06 | 137,07 |
| 7 | Building stone | 129,31 | 129,33 | 130,75 |
| 8 | Electricity materials | 126,68 | 126,68 | 128,35 |
| 9 | Water materials | 126,34 | 126,34 | 127,63 |
| 10 | Architecture r materials | 133,67 | 133,68 | 134,89 |
| 11 | Covering materials | 115,87 | 115,87 | 115,87 |
| II | Manpower |  |  |  |
| 1 | Mason manpower | 234,12 | 234,12 | 234,12 |
| 2 | Carpentering manpower | 234,12 | 234,12 | 234,12 |
| 3 | Steel erecting manpower | 234,12 | 234,12 | 234,12 |
| 4 | Concrete manpower | 234,12 | 234,12 | 234,12 |
| III | Group of construction machines |  |  |  |
| 1 | Group of lifting machines | 138,67 | 138,67 | 138,67 |
| 2 | Group of concrete-related machines | 166,75 | 166,75 | 166,75 |
| 3 | Group of metal processing machines | 158,65 | 158,65 | 158,65 |
| 4 | Group of soil-related machines | 132,17 | 132,17 | 132,17 |
| 5 | Group of transport machines | 141,80 | 141,80 | 141,80 |
| 6 | Group of pile-related machines | 149,54 | 149,54 | 149,54 |