THE SOCIALIST REPUBLIC OF VIETNAM

QCVN 03: 2009/BXD

THE NATIONAL TECHNICAL REGULATION ON CLASSIFICATION, GRADING OF CIVIL AND INDUSTRIAL BUILDINGS AND URBAN INFRASTRUCTURES

HANOI – 2009

PREAMBLE

QCVN 03: 2009/BXD compiled by the Institute of Science and Technology, submitted for approval by the Department of Science, Technology, and Environment, issued by the Ministry of Construction together with Circular No.: 33/2009/TT-BXD dated September 30, 2009.

THE NATIONAL TECHNICAL REGULATION

ON CLASSIFICATION, GRADING OF CIVIL AND INDUSTRIAL BUILDINGS AND URBAN INFRASTRUCTURES

# GENERAL PROVISIONS

## Scope of governing

This Regulation defines types and grades of civil and industrial buildings and urban infrastructure, including buildings of new construction and renovation, repair.

## Subjects of application

This Regulation applies to organizations and detacheds involved in construction activities of civil and industrial buildings and urban infrastructure in the territory of Vietnam.

## Interpretation of terms

In this Regulation the following terms shall be construed as follows:

### Construction buildings

The products are made by human labor power, construction materials and equipment installed into the buildings, which are linked and located to land, which may include the parts in underground, above the ground, below the water surface, above the water surface and built as designed.

### Types of Construction buildings

The conception specifies the use function of the buildings (housing, schools, hospitals, factories producing cement, sypplying water...). An investment project may have many kinds of buildings.

### Grades of buildings

The conception shows the size of the building (height, area, capacity...) or the importance of the building (international, national, provincial grades, grade of rural districts, commune or social-economic consequences, the ecological environment as the building got breakdown.

One type of building can have many different grades. Grade of building is determined for each detached building of the investment project (an apartment, a classroom of schools, a surgery room of the hospital, tower building of heat Exchange of the plant producing cement…).

### Detached housing

The building is built on the premises of the use right of household or detached.

### Villas

Detached houses with gardens (trees, lawns, gardens,...), with fences and separate entrances.

### Apartment

Houses having two floors or more with aisles, stairs and infrastructure systems used for multiple family apartments and detacheds.

### The multi-functional houses (multi--functional combinations)

The building arranged in the same house or room groups, floors with different use function (offices, the auditorium rooms, food services, trade, accommodation and other function rooms.

### The height of the building

Building height from the ground level of the building according to approved plan to the highest point of the building, including tum or slope roof.

NOTE: The technical equipment on the roof: antenna, lightning pole, devices using solar energy, metal water tank ... not included in building height.

### Number of floors

Number of floors of the house include all floors on the ground (including technical floors, attic, tum roof) and the basement half floor.

NOTE: The basements are not counted

### Floors on the ground surface

The floors that their floor level are higher or equal to ground level located the building in accordance with the approved plans.

### Basement

Floor that over half of its height is below ground level located the building according to approved plans.

### Semi-basement floors

Floor that its half of height located above or equal to the ground level of the building according to the approved plans.

### Technical floor

The floor is used for layout of the technical equipment of the building. Technical floor may be a basement, semi-basement floor, attic, or floor of the middle of the house.

### Attic

The floor located within the pitched roof space which the whole or part of its facade is created by the surface of tilt or folding roof, in which vertical wall (if any) is not higher that floor 1.5 m

### Floor area of a floor

Floor area within the outer edge of the wall of the floor. The area of the corridor, balcony, loggia... is also counted in floor area.

### Floor area of the attic

The area measured in floor level within the outer edge of the wall of the attic.

### Total floor areas of the house (the building)

Total floor area of all floors, including basement, semi basement, technical floor, attic and tum roof.

### Urban technical infrastructure

Urban technical infrastructure includes urban transport infrastructure; water supply, drainage, waste water treatment, lighting, green parks, cemeteries, urban waste.

# SPECIFICATION

## Civil buildings

### Classification of civil buildings

Civil buildings are classified as in Table A.1, Appendix A.

#### Housing

- Apartments;

- Detached houses.

#### Public buildings

- Education buildings;

- Health buildings;

- Sports buildings;

- Culture buildings;

- Trade and service buildings;

- Communication and information buildings;

- The airports, waterways, railways, automobiles stations;

- Mixed-used building, hotels, domitories, guest houses, motels;

- Headquarter of state administrative agencies;

- The head office of the business administrative units, enterprises, political-social organizations and other organizations.

### Grading of civil buildings

#### Grading of housing

a) The apartment is divided into four grades (from grade I to IV), detached houses are divided into two grades (grade III and IV) according to Table A.1 of Appendix A.

b) Grade of apartments and detached houses having basements are selected higher grades based on the number of floors above ground (Table A.1) or basements (Table A.2).

FOR EXAMPLE: The 17-floor apartment building has 4 basements: grade of building based on the number of floors on the ground according to Table A.1 is grade II; grade of building based on the number of basements in Table A.2 is grade I. This apartment building is chosen as grade I.

#### Grading of public buildings

- Public Buildings are divided into five grades (special grade and the grades from I to IV) according to Table A.1, Appendix A.

- The public buildings having basements are selected higher grades based on the size (number of floors, area, height) of the above part of the ground surface (Table A.1) or number of basements (Table A.2):

- The public buildings not having in the list in Table A.1 are selected the highest level based on grading criteria according to importance and size of the building as specified in Table A.2;

#### Grading of other buildings of the projects to invest in construction of civil building

a) Industrial buildings (warehouse, manufacturing factories...) are taken by respective industrial building grades specified in Appendix B;

b) The building of urban technical infrastructure (clean water tanks, water pipelines, sewer lines, technical tunnels, infrastructure in urban transport...) is taken by respective technical infrastructure building as specified in Appendix C.

## Industrial buildings

### Classification of Industrial buildings

Industrial buildings are classified as in Table B.1, Appendix B including:

#### Production of construction materials

- Cement production factory;

- Factory producing tiles (Ceramic, granite tiles, ceramic tiles);

- Production factory of bricks, tiles and clay;

- Production factory of sanitary china ware;

- Production factory of glass;

- Production factory of mixed concrete and concrete structures;

- Quarry.

#### The mining building of coal, ore

- Strip coal mine;

- Open-cast coal mine;

- Plant of selection for washing, selection of coal;

- Pit ore mine;

- Open-cast ore mine;

- Plant of selection ore, ore enrichment;

- Natural bauxite mine;

- Alumina production plant.

#### Oil and Gas Industry

- The building of exploration drilling, exploitation on the sea;

- Oil Refinery;

- Gas processing plant;

- Storage of petroleum;

- Storage of liquefied gas;

- Gas and oil pipeline.

#### Heavy Industry

- color metallurgical plant;

- Plant refining, rolling steel;

- The mechanical factory of fabricating dynamics machine and tool machine of all kinds;

- The plant manufacturing industrial equipment and complete equipment;

- Car assembly factory;

- The plant manufacturing assembing motorcycles;

- Energy buildings;

- Chemicals and petrochemicals Industry.

#### Light industry buildings

- Food industry buildings;

- The remaining light industry buildings.

#### Fisheries processing industry

NOTE: The specific types of buildings of 2.2.1.1÷ 2.2.1.6 listed in Table B.1, Appendix B

### Grading of industrial buildings

#### The industrial construction buildings of the line of main production technology is graded according to the scale of production or technological characteristics (capacity, output, contenance, depth of mining, pipeline pressure) into 5 grades (especial level and the grades from grade I to IV) according to Table B.1 of Annex B.

#### The buildings which are in the line of main production technologies that have the underground or structure with the height or big span is further collated with Table B.2 of Annex B and selected the building grade as follows:

- In case of Table B.2, the building having higher grade compared with the specified grade for the buildings which are in the line of main production technologies, the grade of this building is taken according to Table B.2.

FOR EXAMPLE: The height of 105 m heat exchange tower of the cement plant with capacity of 0.9 million ton/year: According to Table B.1 and the building is of grade III; according to Table B.2, the building is of grade II. Grade of the heat exchange tower is chosen as grade II.

- In case of Table B.2, the building having grade less than compared with the specified grade for the buildings which are in the line of main production technologies, the grade of this building is taken according to the building grade of main production line.

FOR EXAMPLE: The cement grinding building of the 24 m largest structures span length of the cement plant with capacity of 1.4 million tons/year: According to Table B.1, the building is of grade II; According to Table B .2, the building is of grade III. Grade of the cement grinding building is chosen as level II.

#### Grading of other buildings of industrial building construction investment projects but is not in the main production line.

- The industrial buildings which are not in the main production technological line, based on the importance of the building or the size of the structure, shall be taken at the grade specified in Table B.2, Appendix B;

- The civil buildings (operating, administrative buildings, canteens ...) are taken according to the respective building grades provided for in Appendix A;

- The technical infrastructure buildings (water supply, wastewater treatment, infrastructure in urban traffic, technical tunnel,...) are taken according to the respective building grades provided for in Appendix C.

## Urban technical infrastructure buildings

### Classification of the urban technical infrastructure buildings

The urban technical infrastructure buildings are classified as in Table C.1, Appendix C, including:

#### Water Supply

#### Drainage

#### Waste treatment

#### Public lighting, parks, urban cemeteries

#### Garages of cars, motorcycles

#### Technical tunnel

#### The infrastructure for urban transport

### Grading of the urban technical infrastructure buildings

#### The technical infrastructure buildings are graded as in Table C.1, Appendix C.

#### The civil buildings under the project of urban technical infrastructure construction are taken according the grades of respective civil buildings provided for in Appendix A.

#### The industrial buildings under the project of urban technical infrastructure construction are taken according the grades of respective industrial buildings provided for in Appendix B.

APPENDIX A

(Provision)

CLASSIFICATION, GRADING OF CIVIL BUILDINGS

Table A.1 Classification, grading of civil buildings

| Type of buildings |  | Grading criteria | Building grade |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Special grade | I | II | III | IV |
| A.1.1 Housing | A.1.1.1 Apartment | Number of floors | - | > 25 | 9 ÷ 25 | 4 ÷ 8 | ≤ 3 |
|  | A.1.1.2 Detached |  | - | - | - | Villas or ≥ 4 | ≤ 3 |
| A.1.2 Public building | A.1.2.1 Educational buildings: nursery, kindergarten, schools at all levels, universities and colleges, professional high schools, vocational schools, schools of technical workers, schools and professional skill schools and other types of schools. | height (m) | - | > 28 | >15÷28 | 6 ÷ 15 | < 6 |
|  | A.1.2.2 Medical buildings: general hospitals, speciality hospitals from central and local areas, general clinics, regional specialty clinics, medical stations, maternity homes, nursing homes, rehabilitation homes, orthopaedic, old people homes, epidemic disease prevention and combat and other health facilities. | height (m) | - | > 28 | >15÷28 | 6 ÷ 15 | < 6 |
|  | A.1.2.3 Sport building: Stadium, sports hall, gymnasium |  |  |  |  |  |  |
|  | Outdoor | Importance or capacity (thousand seats) | International or national grade or > 40 | >20÷40 | 5÷20 | < 5 | - |
|  | Indoor |  | > 7,5 | > 5÷7,5 | 2÷5 | < 2 | - |

Table A.1 classification, grading of civil buildings (continued)

| Type of buildings |  | Grading criteria | Building grade |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Special grade | I | II | III | IV |
| A.1.2 Public buildings | A.1.2.4 Culture building |  |  |  |  |  |  |
|  | a) Convention centers, theaters, cultural houses, clubs, cinemas, circuses | Importance or capacity (thousand seats) | International or national grade or > 3 | > 1,2÷3 | >0,3÷ 1,2 | ≤ 0, 3 | - |
|  | b) Museums, libraries, exhibitions, galleries and other buildings | Importance | International or national grade | provines, branches | remainders | - | - |
|  | c) The relics, religious service | Importance | Special national relics | nation | provines | remainders | - |
|  | d) Recreation and entertainment buildings | Adventure factors or height (m) | - | Adventure games or > 28 m | 15 ÷ 28 | < 15 | - |
|  | A.1.2.5 Commercial and services buildings: trade centers, supermarkets, markets, shops, restaurants, bars... | Total floor area (thousand m2) | > 50 | >15÷50 | > 5÷15 | 0,5 ÷ 5 | < 0,5 |
|  | A.1.2.6 Information, communication buildings |  |  |  |  |  |  |
|  | a) Receiving, broadcasting telecommunications tower, radio and television | height (m) | ≥ 300 | 200 ÷ < 300 | 100 ÷ < 200 | 50 ÷ < 100 | < 50 |
|  | b) The communications service homes (post office, the homes for installation of communication equipment, control tower) | Total floor area (thousand m2) | >15 | > 10÷15 | 5 ÷ 10 | 1 ÷ < 5 | < 1 |
|  | A.1.2.7 Stations: airway, waterway, railway, bus station | Total floor area (thousand m2) | > 50 | >15÷ 50 | > 5÷15 | 0,5 ÷ 5 | < 0,5 |
|  | A.1.2.8 The mix-used buildings, hotels, domitories, guest houses, motels | Number of floors | > 50 | >25÷50 | 9 ÷ 25 | 4 ÷ 8 | ≤ 3 |
|  | A.1.2.9 Headquaters of state administrative agencies: the building of the National Assembly, the Government, the President, Ministries, Branches and Committees at all levels | Importance | Party Central Committee, National Assembly, the President, Government | Provincial Party Committee, and Provincial People's Committees People's Council, Ministries and the equivalent level | District Party Committee, district People's Committees, district People's Council, departments and the equivalent level | Party Committee, commune-level People's Committees, commune-level People's Councils | - |
|  | A.1.2.10 Head offices of the business administrative units, enterprises, social- political organizations and other organizations other than 2.8. | height (m) | - | > 50 | > 28÷50 | > 12÷28 | ≤ 12 |

Table A.2 Grading of civil buildings not listed in Table A.1

| Criteria of grading | Units | Special grade | grade I | grade II | grade III | grade IV |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Importance | Administrtive grade | International, national | Province, centrally-run cities | Rural and urban districts, cities directly under the province | Communes, wards, | - |
| 2. The size of structure |  |  |  |  |  |  |
| a) The height of building | m | - | > 75 | > 28 ÷ 75 | > 12 ÷ 28 | ≤ 12 |
| b) Length of largest structure span | m | - | > 72 | > 36÷72 | 12÷36 | < 12 |
| c) Total floor areas | m2 | - | > 15÷50 | > 5÷15 | 0,5÷5 | < 0,5 |
| d) number of basement | floors | - | 4÷5 | 2÷3 | 1 | - |

APPENDIX B

(Provision)

CLASSIFICATION, GRADING OF THE INDUSTRIAL BUILDINGS

Table B.1 Classification and grading of the industrial buildings of main production technology line

| Type of Buildings |  | Criteria of grading | Grade of Buildings |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | speciality | I | II | III | IV |
| B.1.1 Production of building materials | B.1.1.1 Cement production factory | Capacity (million tons/year) | - | > 2 | 1 ÷ 2 | < 1 | - |
|  | B.1.1.2 production factory of tiles (Ceramic, granite tiles, ceramic tiles) | Capacity (million m2/year) | - | > 5 | 2 ÷ 5 | < 2 | - |
|  | B.1.1.3 Factory production of bricks, clay tiles | Capacity (million bricks/year) | - | > 20 | 10 ÷ 20 | < 10 | - |
|  | B.1.1.4 Production factory of sanitary porcelain | Capacity (million products/ year) | - | - | ≥ 0,5 | < 0,5 | - |
|  | B.1.1.5 Glass production factory | Capacity (million m2/year) | - | > 20 | 10 ÷ 20 | < 10 | - |
|  | B.1.1.6 Production factory of mixed concrete and concrete structures | Capacity (million m3/year) | - | > 1 | 0,5 ÷ 1 | < 0,5 | - |
|  | B.1.1.7 Quarry | Capacity (million m3/year) | - | - | > 3 | 1 ÷ 3 | < 1 |
| B.1.2 The exploitation works of coal, ore | B.1.2.1 Pit coal mine | Output (million tons/year) | - | > 1 | 0,3 ÷ 1 | < 0,3 | - |
|  | B.1.2.2 Open-cast coal mines |  | - | - | > 2 | 0,5 ÷ 2 | < 0,5 |
|  | B.1.2.3 Plant of selection for washing, selection of coal. |  | - | > 5 | 2 ÷ 5 | 0,5÷ < 2 | < 0,5 |
|  | B.1.2.4 Pit ore mines |  | - | > 3 | 1÷ 3 | 0,5 ÷ <1 | < 0,5 |
|  | B.1.2.5 Opencast ore mines |  | - | - | > 2 | 1 ÷ 2 | < 1 |
|  | B.1.2.6 Factory of selecting ore, ore enrichment |  | - | > 7 | 3 ÷ 7 | 1 ÷ < 3 | < 1 |
|  | B.1.2.7 Natural bauxite mines |  | - | > 15 | 7 ÷ 15 | 3 ÷ < 7 | < 3 |
|  | B.1.2.8 Alumina production plant |  | - | > 3 | 1 ÷ 3 | < 1 | - |

Table B.1 Classification and grading of the industrial buildings of main production technology line (Continued)

| Type of Buildings |  | Criteria of grading | Grade of Buildings |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | speciality | I | II | III | IV |
| B.1.3 Oil and Gas Industry | B.1.3.1 Drilling rig of exploration and exploitation on the sea | The depth below sea level (m) | > 5,000 | 300 ÷ 5,000 | < 300 | - | - |
|  | B.1.3.2 Refinery | Capacity (million tons of crude oil/year) | > 10 | 5 ÷ 10 | < 5 | - | - |
|  | B.1.3.3 Gas processing factory | Capacity (million m3 gas/day) | > 10 | 5 ÷ 10 | < 5 | - | - |
|  | B.1.3.4 Petroleum storage | Volume of tank (thoudsand m3) | > 20 | 10 ÷ 20 | 5 ÷< 10 | 1÷< 5 | < 1 |
|  | B.1.3.5 LPG depots | Volume of tank (thoudsand m3) | > 20 | 10 ÷ 20 | 5 ÷< 10 | 1÷< 5 | - |
|  | B.1.3.6 Gas, oil pinelines | Pressure (bar) | Under sea | On shore > 60 | On shore 19 ÷ 60 | On shore 7 ÷< 19 | - |
| B.1.4 Heavyindustry | B.1.4.1 Color metallurgical plant | output (million tons /year) | - | 0,5÷ 1 | 0,1÷ <0,5 | < 0,1 | - |
|  | B.1.4.2 Steel rolling, refining factory | output (million tons /year) | Metallurgical complex | > 1 | 0,5 ÷ 1 | < 0,5 | - |
|  | B.1.4.3 The mechanical factory of manufacturing dynamics and tool machine of all kinds | output (thoudsand pieces /year) | - | > 5 | 2,5 ÷ 5 | < 2,5 | - |
|  | B.1.4.4 The plant manufacturing industrial equipment and complete equipment | output (thoudsand tons /year) | - | > 10 | 5 ÷ 10 | < 5 | - |
|  | B.1.4.5 Automobile assembly plant | output (thoudsand vehicles /year) | - | > 20 | 5 ÷ 20 | < 5 | - |
|  | B.1.4.6 Factory manufacturing, assembling motorcycles | output (million vehicles /year) | - | > 1 | 0,5 ÷ 1 | < 0,5 | - |

Table B.1 Classification and grading of the industrial buildings of main production technology line (Continued)

| Type of Buildings |  | Criteria of grading | Grade of Buildings |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | speciality | I | II | III | IV |
| B.1.4 Heavy industry | B.1.4.7 Energy buildings |  |  |  |  |  |  |
|  | a) Thermal plant (center) | Total capacity (MW) | > 2,000 | 600 ÷ 2,000 | 300 ÷ < 600 | 100 ÷ < 300 | < 100 |
|  | b) Hydropower plant |  | > 1,000 | 300 ÷ 1,000 | 100 ÷ <300 | 50 ÷ < 100 | < 50 |
|  | c) Nuclear power plant |  | All sizes | - | - | - | - |
|  | d) Power lines and substations | Voltage (kV) |  500 | 110 ÷ 220 | 1 ÷ < 110 | < 1 | - |
|  | B.1.4.8 Chemicals and petrochemicals Industry |  |  |  |  |  |  |
|  | Products of fertilizers and plant protection chemicals |  |  |  |  |  |  |
|  | a) Factory manufacturing Urea, DAP, MPA, SA, complex NPK | Output (thoudsand tons /year) | - | > 500 | >200 ÷ 500 | < 200 | - |
|  | b) Phosphate fertilizer production plant of all kinds (superphosphate, melt phosphate) |  | - | > 500 | 300 ÷ 500 | 100 ÷ < 300 | < 100 |
|  | c) Factory producing mixture NPK fertilizer, micro-organisme fertilizer |  | - | - | > 300 | 50 ÷ < 300 | < 50 |
|  | d) Chemical products for plant protection |  |  | > 15 | > 10 ÷ 15 | 3 ÷ 10 | < 3 |
|  | Basic chemical products |  |  |  |  |  |  |
|  | đ) Production plant of ammonia, acid, caustic soda, chlorine of all types | Output (thoudsand tons /year) | > 200 | 100 ÷ 200 | 40 ÷ < 100 | < 40 | - |
|  | e) Soda production factory |  |  | > 300 | 200 ÷ 300 | < 200 | - |
|  | f) Factory producing inorganic salts, inorganic oxide |  | - | - | > 20 | 10 ÷ 20 | < 10 |
|  | g) Factory producing pure and clean inorganic chemicals |  | - | > 20 | 10 ÷ 20 | < 10 | - |

Table B.1 Classification and grading of the industrial buildings of main production technology line (Continued)

| Type of Buildings |  | Criteria of grading | Grade of Buildings |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | speciality | I | II | III | IV |
| B.1.4. Heavy industry | B.1.4.8 Chemicals and petrochemicals industry |  |  |  |  |  |  |
|  | Manufacture of rubber and cleaning products |  |  |  |  |  |  |
|  | h) Factory manufacturing automobile tires - tractor (Standard of Size 900-20) | Output (million piece/year) | - | > 1 | 0, 5 ÷ 1,0 | 0,2 ÷ < 0,5 | - |
|  | i) Factory manufacturing tires of motorcycles, bicycles |  | - | - | > 5 | 1÷5 | < 1 |
|  | k) Convoyer manufacture factory | Output (thousand m2/year) | - | - | > 500 | 200÷ 500 | < 200 |
|  | l) Factory producing technical rubber | Output (million products/year) | - | - | > 1,5 | 0,5 ÷ 1,5 | < 0,5 |
|  | m) Factory producing cleaning products (detergents, washing powder, shampoo, washing soap, soap ...) | Output (thoudsand tons /year) | - | - | 15 ÷ 30 | 5 ÷ < 15 | < 5 |
|  | Products of electrochemistry, paints, chemicals mine materials |  |  |  |  |  |  |
|  | n) Cell production factory | Output (million pieces/year) | - | - | > 150 | 15 ÷ 150 | < 15 |
|  | o) Battery production factory | Output (thousand Kwh/year) | - | > 450 | 150 ÷ 450 | < 150 | - |
|  | p) Factory producing paints of all kinds, alkyd resin, acrylic materials | Output (thoudsand tons /year) | - | - | - | > 5 ÷ 20 | < 5 |
|  | q) Factory producing chemical mine materials (Appetite ore selection) | Output (thoudsand tons /year) | - | > 600 | 350 ÷ 600 | 100 ÷ < 350 | - |
|  | Petrochemicals and other chemicals |  |  |  |  |  |  |
|  | r) Petrochemical production plant (PP, PE, PVC, PS, PET, SV, fiber, DOP, polystyrene, LAB, synthetic rubber) | Output (thoudsand tons /year) | > 500 | 200 ÷ 500 | < 200 | - | - |

Table B.1 Classification and grading of the industrial buildings of main production technology line (Continued)

| Type of Buildings |  | Criteria of grading | Grade of Buildings |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | speciality | I | II | III | IV |
| B.1.4. Heavy industry | B.1.4.8 Chemicals and petrochemicals industry |  |  |  |  |  |  |
|  | s) Indistrial gas production factory | Output (thoudsand m3/h) | - | - | > 8,5 | 3,0 ÷ 8,5 | < 3,0 |
|  | t) Production factory of welding, tire-hoop, steel wire | Output (thoudsand tons /year) | - | - | - | ≥ 3,0 | < 3,0 |
|  | u) Pharmaceutical production plant (micro), drugs | Size | All sizes | - | - | - | - |
|  | v) Factory producing chemicals, explosives | Size | All sizes | - | - | - | - |
|  | w) Cosmetic production factory | Output (thoudsand tons /year) | - | - | > 5,0 | < 5,0 | - |
| B.1.5. Light industry | B.1.5.1 Food |  |  |  |  |  |  |
|  | a) Milk factory | Output (million litres/year) | - | ≥100 | 30 ÷ <100 | <30 | - |
|  | b) Factory producing confectionery, instant noodle | Output (thoudsand tons /year) | - | > 25 | 5 ÷ 25 | < 5 | - |
|  | c) Cold storage | Capacity (thoudsand tons) | - | > 1 | 0,25 ÷ ≤1 | < 0,25 | - |
|  | d) Production plant of oils, aromatic | Output (million litres/year) | - | ≥ 150 | 50 ÷ <150 | < 50 | - |
|  | e) Production plant of winery, beer, soft drinks | Output (million litres/year) | - | ≥ 100 | 25 ÷ <100 | < 25 | - |
|  | f) Cigarette production factory | Output (million bags/year) | - | ≥ 200 | 50 ÷ <200 | < 50 | - |

Table B.1 Grading of the buildings of main production technology line (Continued)

| Type of Buildings |  | Criteria of grading | Grade of Buildings |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | speciality | I | II | III | IV |
| B.1.5 Light industry | B.1.5.2 The remaining buildings |  |  |  |  |  |  |
|  | a) Weaving factory | Output (million meters/year) | - | ≥ 25 | 5 ÷ < 25 | < 5 | - |
|  | b) Printing, dying factory | Output (million meters/year) | - | ≥ 35 | 10 ÷ < 35 | < 10 | - |
|  | c) Production plant of sewing products | Capacity (million products/year) | - | ≥ 10 | 2 ÷ <10 | < 2 | - |
|  | d) Tanneries and production factory of leather products | Output (million products/year) | - | ≥ 12 | 1 ÷ <12 | < 1 | - |
|  | e) Production factory of plastic products | Output (thoudsand tons /year) | - | ≥ 15 | 2 ÷ <15 | < 2 | - |
|  | f) Production factory of crockery, glass |  | - | ≥ 25 | 3 ÷ < 25 | < 3 | - |
|  | g) Pulp and paper production plant |  | - | ≥ 60 | 25 ÷ < 60 | < 25 | - |
|  | i) Factory assemblin electronics (televisions, computers and equivelant products), refrigeration (air conditioner, refrigerators and similar products) | Output (thousand products/year) | - | > 300 | 100 ÷ 300 | < 100 | - |
|  | k) The factory manufacturing parts, information accessories and electronics (electronic printed circuit boards, IC and similar products) | Output (million products/year) | - | ≥ 400 | 300 ÷ <400 | 200 ÷ <300 | < 200 |
|  | l) Cigarette production factory | Output (million bags/year) | - | ≥ 200 | 50 ÷ <200 | < 50 | - |
| B.1.6 Fisheries processing industry | Fisheries processing factory | Output (ton of material /day) | - | > 300 | 100 ÷ 300 | 20 ÷ 100 | <20 |

Table B.2 Grading of the industrial buildings according to the importance or size of the structure

| Criteria of grading | Units | Special grade | grade I | grade II | grade III | grade IV |
| --- | --- | --- | --- | --- | --- | --- |
| 1. Importance |  |  |  |  |  |  |
| a) Repository for radioactive sources; | The consequences of economy, socety and ecological environment | All sizes | - | - | - | - |
| b) Manufacturers, warehouses of explosives |  | All sizes | - | - | - | - |
| 2. Size of structure |  |  |  |  |  |  |
| a) Heigh | (m) | - | >200 | >100 ÷ 200 | >50÷ 100 | ≤50 |
| b) Length of largest structure span | (m) | - | >72 | >36 ÷72 | 12÷36 | <12 |
| c) Number of the basements or the depth of underground part | Number of the basements or (m) | - | ≥ 4 basements or >12m | 2÷3 basements or 6÷12 m | 1 basements or 3÷<6 m | - |

APPENDIX C

(Provision)

CLASSIFICATION, GRADING OF THE URBAN TECHNICAL INFRASTRUCTURE BUILDINGS

Table C.1 Classification and grading of the urban technical infrastructure buildings

| Type of Buildings |  | Criteria of grading | Grade of Buildings |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | speciality | I | II | III | IV |
| C.1.1 Water supply | C.1.1.1 The exploitation building of raw water, raw water pumping stations, water treatment facilities, water pumping stations | thousand m3/ngđ | - | > 100 | 30 ÷ 100 | 5 ÷ <30 | < 5 |
|  | C.1.1.2 Clean wáter tanks | Thousand m3 | - | ≥ 10 | 5 ÷ <10 | 1 ÷ <5 | < 1 |
|  | C.1.1.3 Water supply pipeline | Pipe diameter (mm) | - | ≥ 1,200 | 800 ÷ < 1,200 | 300 ÷ < 800 | < 300 |
|  | C.1.1.4 Water Station | Thousand m3 | - | ≥ 2 | 1 ÷ <2 | 0,1 ÷ <1 | < 0,1 |
| C.1.2 drainage | C.1.2.1 Rainwater drainage culverts, drainage culverts of waste water, the public culverts | Pipe diameter (mm) or equivalent cross section | - | ≥ 2,000 | 1.200 ÷ < 2,000 | 700 ÷ <1,200 | < 700 |
|  | C.1.2.2 Regulating reservoirs | ha | - | ≥ 20 | 15 ÷ <20 | 5 ÷ < 15 | < 5 |
|  | C.1.2.3 rainwater pumping station | m3/s | - | ≥ 25 | 10 ÷ <25 | 2 ÷ < 10 | < 2 |
|  | C.1.2.4 Waste water pumping stations, treatment buildings | Thoudsand m3/ngd | - | ≥ 100 | 30 ÷ <100 | 5 ÷ < 30 | < 5 |
|  | C.1.2.5 Sludge treatment buildings | m3/ngd | - | ≥ 1,000 | 200 ÷ < 1,000 | 50 ÷ < 200 | < 50 |
| C.1.3 Waste treatment | C.1.3.1 Municipal solid waste |  |  |  |  |  |  |
|  | a) Landfills | ton/ngd | - | ≥ 1,000 | 200 ÷ < 1,000 | 50 ÷ < 200 | < 50 |
|  | b) Plant of combustion, waste treatment and processing | ton/ngd | - | ≥ 500 | 100 ÷ < 500 | 25 ÷ < 100 | < 25 |
|  | C.1.3.2 Hazardous solid waste | ton/ngd | - | ≥ 100 | 20 ÷ < 100 | < 20 | - |
| C.1.4 Public lighting |  | Urban scale | - | - | - | Grade-3 cities or higher | remainders |
| C.1.5 Green tres park |  | Urban scale | - | - | - | -3 cities or higher | remainders |
| C.1.6 Urban Cemeteries |  |  | - | - | - | -3 cities or higher | remainders |

Table C.1 grading of the urban technical infrastructure buildings (the end)

| Type of Buildings |  | Criteria of grading | Grade of Buildings |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | speciality | I | II | III | IV |
| C.1.7 Garage of automobiles and motorcycles (1) | C.1.5.1 Underground garage | Number of basements or the depth (m) | - | 4 ÷ 5 or 12 m ÷18 m | 2 ÷ 3 or 6 m ÷ < 12 m | 1 or < 6 m | - |
|  | C.1.5.2 Floated garage | Number of basements or the height (m) | - | - | 6 ÷ 9 or 18 m ÷ 32,6 m | 3 ÷ 5 or 9 m ÷ < 18 m | ≤ 2 or < 9 m |
| C.1.8 Technical tunnels (tunnel containing power cables, communication cables, water supply pipes…) |  | Tunnel diameter (mm) or equivalent cross section | - | ≥ 2,000 | 1,200 ÷ < 2,000 | 700 ÷ < 1,200 | < 700 |
| The infrastructure for urban transport |  |  |  |  |  |  |  |
| C.1.9 Metro building |  | Size | All sizes | - | - | - | - |
| C.1.10 Motorways, urban roads |  | Converted vehicles traffic/ day | > 30,000 | 10,000 ÷ <30,000 | 3,000 ÷ <10,000 | 300 ÷ <3,000 | < 300 |
| C.1.11 Railways |  |  | High speed railway | Aerial railway | National railway | Specialized railway | - |
| C.1.12 Bridges, pedestrian bridges, railway bridges |  | Span (m) | - | > 100 | 50 ÷ 100 | 25 ÷ 50 | < 25 |
| C.1.13 Tunnels for automobiles, railway, pedestrian |  | Length (m) | - | > 1,000 | 100 ÷ 1,000 | 25 ÷ < 100 | < 25 |
| C.1.14 Waterways |  |  |  |  |  |  |  |
| - Wharf, building dock |  | Capacity (DWT) | - | > 50,000 | 30,000 ÷ 50,000 | 10,000 ÷ < 30,000 | < 10,000 |
| - Ports and berths for ships, shipyards, ship repair factory |  | Tons | - | > 3,000 | 1,500 ÷ 3,000 | 750 ÷ < 1,500 | < 750 |
| - Navigation lock |  |  | - | > 1,500 | 750 ÷ 1,500 | 200 ÷ < 750 | < 200 |
| - River traffic |  | Width B, depth H of water ballast (m) | - | B ≥ 90 H ≥ 4 | B = 70 ÷ < 90 H = 3 ÷ < 4 | B = 50 ÷ < 70 H = 2 ÷ <3 | B < 50 H < 2 |
| C.1.15 Runaway |  | ICAO | IV E | IV D | III C | II B | I A |
| NOTE 1: - Garages of cars and motorcycles with both the supernatant and the underground located in other buildings, the grade of buildings is taken according to the higher grade. - Grade of garage is taken according to the higher grade when determining by the both criterias of number of floors and depth (height). |  |  |  |  |  |  |  |