SOCIALIST REPUBLIC OF VIETNAM

QCVN 19: 2009/BTNMT

NATIONAL TECHNICAL REGULATION

ON INDUSTRIAL EMISSION FOR DUSTS AND INORGANIC SUBSTANCES

HANOI - 2009

Preamble
QCVN 19: 2009/BTNMT compiled by the drafting committee of national technical regulations on air quality, General Department of Environment, Department of Science and Technology, and the Department of Legal Affairs submit for approval and issue together with the Circular No.25/2009/TT-BTNMT dated November 16, 2009 of Ministry of Natural Resources and Environment.

NATIONAL TECHNICAL REGULATION

ON INDUSTRIAL EMISSION FOR DUSTS AND INORGANIC SUBSTANCES

1. GENERAL PROVISIONS

1.1. Scope of governing

This Regulation specifies maximum allowable concentrations of dusts and other inorganic substances in industrial emission as released into the atmosphere.

1.2. Subjects of application

This Regulation applies to organizations and individuals related to the operation of industrial waste gas emissions containing dusts and inorganic substances into the atmosphere.

Emissions of some industrial branches and specific areas of activity are specified separately.

1.3. Interpretation of terms

In this Regulation, the following terms are construed as follows:

1.3.1. Industrial emissions are a mixture of material composition emitted into atmosphere from smokestacks, exhaust pipes of the establishments of production, processing, trade and services of industry.

1.3.2. Dusts are the small solid particles, typically as the particles smaller than 75 mm in diameter, self-deposited by their weight but still are able to hover for a period of time. [according to TCVN 5966:2009 (ISO 4225-1994)

1.3.3. Standard cubic meter of waste gas (Nm3) means a cubic meter of exhaust gas at a temperature of 250C and an absolute pressure of 760 mm of mercury.

1.3.4. Kp means the coefficient of waste flow corresponding to total flow of waste gas of the establishments of production, processing, trade and industrial services.

1.3.5. Kv means the coefficient of the region, area corresponding to locations of factories, the establishments of production, processing, trade and industrial services emitting waste gas into the atmosphere.

1.3.6. P (m3/h) means the total flow of the waste gas of smokestacks, exhaust pipes of the establishments of production, processing, trade and services of industry.

2. TECHNICAL PROVISIONS

2.1. Maximum allowable concentrations of dusts and other inorganic substances in industrial emission are calculated as follows:

Cmax = C x Kp x Kv

Of which:

- Cmax means the maximum allowable concentration of dusts and other inorganic substances in industrial emission, measured in milligrams per cubic meter of standard waste gas (mg/ Nm3);

- C is the concentration of dusts and other inorganic substances specified in Section 2.2;

- Kp is the waste discharge flow coefficient defined in Section 2.3;

- Kv is the coefficient of region or area defined in Section 2.4.

2.2. C concentration of dusts and other inorganic substances as basis for calculating the maximum allowable concentrations in industrial emissions is defined in Table 1 below:

Table 1 - Concentration C of dusts and other inorganic substances as basis for calculating the maximum allowable concentrations in industrial emissions

| No. | Parameters | Concentration C (mg/Nm3) |  |
| --- | --- | --- | --- |
|  |  | A | B |
| 1 | Total dust | 400 | 200 |
| 2 | Silica-containing dust | 50 | 50 |
| 3 | Ammonia and ammonium compounds | 76 | 50 |
| 4 | Antimony and its compounds, calculated as Sb | 20 | 10 |
| 5 | Arsenic and its compounds, calculated as As | 20 | 10 |
| 6 | Cadmium and its compounds, calculated as Cd | 20 | 5 |
| 7 | Lead and its compounds, calculated as Pb | 10 | 5 |
| 8 | Carbon oxides, CO | 1000 | 1000 |
| 9 | Chlorine | 32 | 10 |
| 10 | Copper and its compounds, calculated as Cu | 20 | 10 |
| 11 | Zinc and compounds, calculated as Zn | 30 | 30 |
| 12 | Hydrochloric acid, HCl | 200 | 50 |
| 13 | Flo, HF, or inorganic compounds of fluorine, calculated as HF | 50 | 20 |
| 14 | Hydrogen sulfide, H2S | 7.5 | 7.5 |
| 15 | Sulfur dioxide, SO2 | 1500 | 500 |
| 16 | Nitrogen oxides, NOx (calculated as NO2) | 1000 | 850 |
| 17 | Nitrogen oxides, NOx (chemicals production establishments), calculated as NO2 | 2000 | 1000 |
| 18 | H2SO4 vapor or SO3, calculated SO3 | 100 | 50 |
| 19 | HNO3 vapor (other sources), calculated as NO2 | 1000 | 500 |

Of which:

- Column A specifies C concentration of dusts and other inorganic substances as a basis for calculating the maximum allowable concentrations of industrial emissions for the establishments of production, processing, trade and service of industry operating before January 16, 2007 with the period of application to December 31, 2014;

- Column B specifies C concentrations of dusts and other inorganic substances as a basis for calculating the maximum allowable value of industrial emissions for:

+ The establishments of production, processing, trade and service of industry operating before January 16, 2007;

+ All establishments of production, processing, trade and service of industry with application time from January 01, 2015.

2.3. Waste discharge flow coefficient Kp is specified in Table 2 below:

Table 2: KP waste discharge flow coefficient

| Waste discharge flow (m3/h) | Kp coefficient |
| --- | --- |
| P ≤ 20,000 | 1 |
| 20,000 < P ≤ 100,000 | 0.9 |
| P>100,000 | 0.8 |

2.4. Kv coefficient of region or area is specified in Table 3 below:

Table 3: Kv coefficient of region or area

| Allocation of region or area |  | Kv coefficient |
| --- | --- | --- |
| Type 1 | Inner city of urban areas of special type (1) and type I (1) specially-used forests (2); natural heritage, cultural historical relics ranked (3); establishments of manufacturing industrially, processing, trade, services and other industrial activities having the distance to the boundary of these areas under 02 km. | 0.6 |
| Type 2 | Inner city of urban areas of type II, III, IV (1); suburbs of urban areas of special type, type I having the distance to the inner city boundary greater than or equal to 02 km; establishments of manufacturing industrially, processing, trade, services and other industrial activities having the distance to the boundary of these areas under 02 km. | 0.8 |
| Type 3 | Industrial zones, urban areas of type V (1); suburbs of urban areas of type II, III, IV having a distance to the boundary of the inner city greater than or equal to 02 km; the establishments of manufacturing industrially, processing, trade, services and other industrial activities having the distance to the boundary of these areas under 02 km (4). | 1.0 |
| Type 4 | Rural areas | 1.2 |
| Type 5 | Mountainous, rural areas | 1.4 |
| Notes: (1) Urban areas defined according to provisions in the Decree No.42/2009/ND-CP dated May 07, 2009 by the Government on the classification of urban areas; (2) Specially-used forests defined by the Law on Protection and Development of Forests dated December 14, 2004 including national parks; natural reserving areas; landscape protected areas; forest for research, experiments of science; (3) Natural heritage, cultural, historical relics ranked by UNESCO, the Prime Minister or the governing ministry; (4) Where the emission sources having a distance to 02 areas or more smaller than 02 km, apply a Kv coefficient of region, area for the areas with the smallest coefficient; (5) The distance specified in Table 3 is calculated from emission sources. |  |  |

3. METHOD OF DETERMINATION

3.1. Method of determining the concentration of dusts and other inorganic substances in industrial emissions in the establishments of manufacturing industrially, processing, trade, services and other industrial activities shall comply with the following national standards:

- TCVN 5977:2005 Emissions of static source - Determination of the value and flow of dusts in the air pipes - Manual volume method;

- TCVN 6750:2005 Emissions of static source - Determination of mass concentration of sulfur dioxide - ion gas chromatographic method;

- TCVN 7172:2002 Emissions of static source - Determination of mass concentration of nitrogen oxides - Photometric method using naphtyletylendiamin;

- TCVN 7242:2003 medical waste incinerators. Method of determining the concentration of carbon monoxide (CO) in emissions;

- TCVN 7243:2003 medical waste incinerators. Method of determining the concentration of hydrofluoric acid (HF) in emissions;

- TCVN 7244:2003 medical waste incinerators. Method of determining the concentration of hydrochloric acid (HCl) in emissions;

3.2. When there are no national standards to determine the concentration of inorganic substances in industrial emissions specified in this Regulation, apply international standards with equivalent accuracy or higher.

4. ORGANIZATION OF IMPLEMENTATION

4.1. This Regulation replaces the application of Vietnam standards TCVN 5939:2005 on Air Quality - Industrial emission standards for dusts and other inorganic substances issued together with the Decision No.22/2006/QD- BTNMT dated December 18, 2006 of the Minister of Natural Resources and Environment on the compulsory application of Vietnam standards on the environment.

4.2. State management agencies on environment are responsible for guiding, inspecting and supervising the implementation of this Regulation.

4.3. Where the national standards on methods of determination referred to in Section

3.1 of this Regulation are amended, supplemented or replaced, the new standards shall be applied.