SOCIALIST REPUBLIC OF VIETNAM

QCVN 20: 2009/BTNMT

NATIONAL TECHNICAL REGULATION ON INDUSTRIAL EMISSION FOR ORGANIC SUBSTANCES

HANOI - 2009

Preamble
QCVN 20: 2009/BTNMT compiled by the drafting committee of national technical regulations on air quality, General Department of Environment, Department of Science and Technology, and the Department of Legal Affairs submit for approval and issue together with the Circular No.25/2009/TT-BTNMT dated November 16, 2009 of Ministry of Natural Resources and Environment

NATIONAL TECHNICAL REGULATION

ON INDUSTRIAL EMISSION FOR ORGANIC SUBSTANCES

1. GENERAL PROVISIONS

1.1. Scope of governing

This Regulation specifies maximum allowable concentrations of organic substances in industrial emission as released into the atmosphere.

1.2. Subjects of application

This Regulation applies to organizations and individuals related to the operation of industrial waste gas emissions containing organic substances into the atmosphere. 
Emissions of some industrial branches and specific areas of activity are specified separately.

1.3. Interpretation of terms

In this regulation, the following terms are construed as follows:

1.3.1. Industrial emission is a mixture of material composition emitted into atmosphere from smokestacks, exhaust pipes of the establishments of production, processing, trade and services of industry.

1.3.2. Standard cubic meter of waste gas (Nm3) means a cubic meter of exhaust gas at a temperature of 250C and an absolute pressure of 760 mm of mercury.

2. TECHNICAL PROVISIONS

Maximum allowable concentrations of certain organic substances in industrial waste gas emitted into the atmosphere are specified in Table 1 below:

Table 1 - Maximum allowable concentration of organic substances in industrial waste gas emitted into the atmosphere

| No. | Name | CAS number | Chemical formula | Maximum concentration (mg/Nm3) |
| --- | --- | --- | --- | --- |
| 1 | Acetylene tetrabromide | 79-27-6 | CHBr2CHBr2 | 14 |
| 2 | Acetaldehyde | 75-07-0 | CH3CHO | 270 |
| 3 | Acrolein | 107-02-8 | CH2=CHCHO | 2,5 |
| 4 | Amyl acetate | 628-63-7 | CH3COOC5H11 | 525 |
| 5 | aniline | 62-53-3 | C6H5NH2 | 19 |
| 6 | Benzidine | 92-87-5 | NH2C6H4C6H4NH2 | KPHĐ |
| 7 | Benzene | 71-43-2 | C6H6 | 5 |
| 8 | benzyl chloride | 100-44-7 | C6H5CH2CI | 5 |
| 9 | 1,3-Butadiene | 106-99-0 | C4H6 | 2200 |
| 10 | n- butyl acetate | 123-86-4 | CH3COOC4H9 | 950 |
| 11 | Butylamine | 109-73-9 | CH3(CH2)2CH2NH2 | 15 |
| 12 | Cresol | 1319-77-3 | CH3C6H4OH | 22 |
| 13 | Chlorobenzene | 108-90-7 | C6H5CI | 350 |
| 14 | Chlorofom | 67-66-3 | CHCI3 | 240 |
| 15 | ß-chlopren | 126-99-8 | CH2=CCICH=CH2 | 90 |
| 16 | Chloropicrin | 76-06-2 | CCI3NO2 | 0,7 |
| 17 | Cyclohexane | 110-82-7 | C6H12 | 1300 |
| 18 | Cyclohexanole | 108-93-0 | C6H11OH | 410 |
| 19 | Cyclohexanone | 108-94-1 | C6H10O | 400 |
| 20 | Cyclohexen | 110-83-8 | C6H10 | 1350 |
| 21 | Diethylamine | 109-89-7 | (C2H5)2NH | 75 |
| 22 | Diflourodibromomethane | 75-61-6 | CF2Br2 | 860 |
| 23 | o-dichlorobenzene | 95-50-1 | C6H4CI2 | 300 |
| 24 | 1,1-Dicloroethane | 75-34-3 | CHCI2CH3 | 400 |
| 25 | 1,2-Dichloroethylene | 540-59-0 | CICH=CHCI | 790 |
| 26 | 1,4-Dioxan | 123-91-1 | C4H8O2 | 360 |
| 27 | Dimethylaniline | 121-69-7 | C6H5N(CH3)2 | 25 |
| 28 | Dicloroethyl ether | 111-44-4 | (CICH2CH2)2O | 90 |
| 29 | Dimethylfomamide | 68-12-2 | (CH3)2NOCH | 60 |
| 30 | Dimethyl sulfate | 77-78-1 | (CH3)2SO4 | 0,5 |
| 31 | Dimethylhydrazine | 57-14-7 | (CH3)2NNH2 | 1 |
| 32 | Dinitrobobenzene | 25154-54-5 | C6H4(NO2)2 | 1 |
| 33 | Ethyl acetate | 141-78-6 | CH3COOC2H5 | 1400 |
| 34 | Ethyl amine | 75-04-7 | CH3CH2NH2 | 45 |
| 35 | Ethyl benzene | 100-41-4 | CH3CH2C6H5 | 870 |
| 36 | ethyl bromide | 74-96-4 | C2H5Br | 890 |
| 37 | Ethylene diamine | 107-15-3 | NH2CH2CH2NH2 | 30 |
| 38 | Ethylene dibromua | 106-93-4 | CHBr=CHBr | 190 |
| 39 | methyl acrylates | 140-88-5 | CH2=CHCOOC2H5 | 100 |
| 40 | Ethylene clohydrin | 107-07-3 | CH2CICH2OH | 16 |
| 41 | Ethylene oxide | 75-21-8 | CH2OCH2 | 20 |
| 42 | Ethyl ether | 60-29-7 | C2H5OC2H5 | 1200 |
| 43 | Ethyl cloride | 75-00-3 | CH3CH2CI | 2600 |
| 44 | Ethyl silicate | 78-10-4 | (C2H5)4SiO4 | 850 |
| 45 | Ethanol amine | 141-43-5 | NH2CH2CH2OH | 45 |
| 46 | Furandehyde | 98-01-1 | C4H3OCHO | 20 |
| 47 | Formaldehyde | 50-00-0 | HCHO | 20 |
| 48 | Furfuryl (2-Furylmethanol) | 98-00-0 | C4H3OCH2OH | 120 |
| 49 | Flourotrichloromethane | 75-69-4 | CCI3F | 5600 |
| 50 | n-Heptane | 142-82-5 | C7H16 | 2000 |
| 51 | n-Hexane | 110-54-3 | C6H14 | 450 |
| 52 | Isopropylamine | 75-31-0 | (CH3)2CHNH2 | 12 |
| 53 | n-buthanol | 71-36-3 | CH3(CH2)3OH | 360 |
| 54 | Methyl mercaptan | 74-93-1 | CH3SH | 15 |
| 55 | Methyl acetate | 79-20-9 | CH3COOCH3 | 610 |
| 56 | Methyl acrylate | 96-33-3 | CH2=CHCOOCH3 | 35 |
| 57 | Methanol | 67-56-1 | CH3OH | 260 |
| 58 | Methyl acethylene | 74-99-7 | CH3C=CH | 1650 |
| 59 | Methyl bromide | 74-83-9 | CH3Br | 80 |
| 60 | Methyl cychlohecxane | 108-87-2 | CH3C6H11 | 2000 |
| 61 | Methyl cychlohecxanol | 25639-42-3 | CH3C6H10OH | 470 |
| 62 | Methyl cychlohecxanone | 1331-22-2 | CH3C6H9O | 460 |
| 63 | Methyl cloride | 74-87-3 | CH3CI | 210 |
| 64 | Methylene cloride | 75-09-2 | CH2CI2 | 1750 |
| 65 | Methyl cloroform | 71-55-6 | CH3CCI3 | 2700 |
| 66 | Monomethylaniline | 100-61-8 | C6H5NHCH3 | 9 |
| 67 | Methanol amine | 3088-27-5 | HOCH2NH2 | 31 |
| 68 | Naphthalene | 91-20-3 | C10H8 | 150 |
| 69 | Nitrobenzene | 98-95-3 | C6H5NO2 | 5 |
| 70 | Nitroethane | 79-24-3 | CH3CH2NO2 | 310 |
| 71 | Nitroglycerin | 55-63-0 | C3H5(ONO2)3 | 5 |
| 72 | Nitromethane | 75-52-5 | CH3NO2 | 250 |
| 73 | 2-Nitropropane | 79-46-9 | CH3CH(NO2)CH3 | 1800 |
| 74 | Nitrotoluene | 1321-12-6 | NO2C6H4CH3 | 30 |
| 75 | 2-Pentanon | 107-87-9 | CH3CO(CH2)2CH3 | 700 |
| 76 | Phenol | 108-95-2 | C6H5OH | 19 |
| 77 | Phenyl hydrazine | 100-63-0 | C6H5NHNH2 | 22 |
| 78 | n-Propanol | 71-23-8 | CH3CH2CH2OH | 980 |
| 79 | n-Propylacetate | 109-60-4 | CH3-COO-C3H7 | 840 |
| 80 | Propylenedicloride | 78-87-5 | CH3-CHCI-CH2CI | 350 |
| 81 | Propylene oxyde | 75-56-9 | C3H6O | 240 |
| 82 | Pyridine | 110-86-1 | C5H5N | 30 |
| 83 | Pyrene | 129-00-o | C16H10 | 15 |
| 84 | p-Quinol | 106-51-4 | C6H4O2 | 0,4 |
| 85 | Styrene | 100-42-5 | C6H5CH=CH2 | 100 |
| 86 | Tetrahydrofural | 109-99-9 | C4H8O | 590 |
| 87 | 1,1,2,2-Tetrachloroethane | 79-34-5 | CI2HCCHCI2 | 35 |
| 88 | Tetrachloethylene | 127-18-4 | CCI2=CCI2 | 670 |
| 89 | Tetrachlomethane | 56-23-5 | CCI4 | 65 |
| 90 | Tetranitromethane | 509-14-8 | C(NO2)4 | 8 |
| 91 | Toluene | 108-88-3 | C6H5CH3 | 750 |
| 92 | 0-Toluidine | 95-53-4 | CH3C6H4NH2 | 22 |
| 93 | Toluene-2,4-diisocyanate | 584-84-9 | CH3C6H3(NCO)2 | 0,7 |
| 94 | Triethyl amine | 121-44-8 | (C2H5)3N | 100 |
| 95 | 1,1,2-Trichloroethane | 79-00-5 | CHCI2CH2CI | 1080 |
| 96 | Tricloroethylene | 79-01-6 | CICH=CCI2 | 110 |
| 97 | Xylene | 1330-20-7 | C6H4(CH3)2 | 870 |
| 98 | Xylidine | 1300-73-8 | (CH3)2C6H3NH2 | 50 |
| 99 | Vinyl cloride | 75-01-4 | CH2=CHCI | 20 |
| 100 | Vinyltoluene | 25013-15-4 | CH2=CHC6H4CH3 | 480 |
| Notes: - CAS number: Number of international registration of chemicals (Chemical Abstracts Service Registry Number); - KPHD is undetectable. |  |  |  |  |

3. METHOD OF DETERMINIATION

3.1. Method of determining the concentration of organic substances in industrial emissions shall comply with current national standards.

3.2. When there are no national standards to determine the concentration of organic substances in industrial emissions specified in this Regulation, apply international standards with equivalent accuracy or higher.

4. ORGANIZATION OF IMPLEMENTATION

4.1. This Regulation replaces the application of Vietnam standards TCVN 5939:2005 on Air Quality - Industrial emission standards for dusts and organic substances issued together with the Decision No.22/2006/QD- BTNMT dated December 18, 2006 of the Minister of Natural Resources and Environment on the compulsory application of Vietnam standards on the environment.

4.2. State management agencies on environment are responsible for guiding, inspecting and supervising the implementation of this Regulation.