SOCIALIST REPUBLIC OF VIETNAM

##### QCVN 25: 2009/BTNMT

NATIONAL TECHNICAL REGULATION ON WASTE WATER 
OF THE SOLID WASTE LANDFILL

HANOI - 2009

Preamble

QCVN 25: 2009/BTNMT compiled by the compilation committee of national technical regulations on water quality, the General Department of Environment, Department of Science and Technology, Department of Legal Affairs submit for approval and issued under the Circular No.25/2009/TT-BTNMT dated November 16, 2009 of the Ministry of Natural Resources and Environment.

NATIONAL TECHNICAL REGULATION ON WASTEWATER OF THE SOLID WASTE LANDFILLS

1.GENERAL PROVISIONS

1.1. Scope of governing

This Regulation stipulates the maximum allowable concentration of the pollution parameters in wastewater of the solid waste landfills when discharged into the receiving source.

1.2. Application subjects

This Regulation applies to all organizations and individuals involved in solid waste burying activity.

1.3. Interpretation of terms

In this Regulation, the following terms shall be construed as follows:

1.3.1. Solid waste landfill is the location to conduct the solid waste disposal by burial method.

1.3.2. Waste water of solid waste landfill is the solution of waste from the landfills into the receiving source.

1.3.3. Waste water receiving source is surface water source or coastal water zone with determined purposes, place where wastewater of solid waste landfills discharged into.

2. TECHNICAL REGULATIONS

2.1. The maximum allowable concentration of pollution parameters in wastewater of the solid waste landfills discharged into the receiving source specified in Table 1 below:

Table 1: The maximum allowable concentration of pollution parameters in wastewater of the solid waste landfills

| No. | Parameters | Maximum allowable concentration (mg/l) |  |  |
| --- | --- | --- | --- | --- |
|  |  | A | # B1 | # B2 |
| 1 | BOD5 (20 oC) | 30 | 100 | 50 |
| 2 | COD | 50 | 400 | 300 |
| 3 | Total Nitrogen | 15 | 60 | 60 |
| 4 | Ammonium, by N | 5 | 25 | 25 |

In which:

- Column A specifies maximum allowable concentration of the pollution parameters in wastewater of the solid waste landfills when discharged into water sources used for domestic water supply purpose;

- Column B1 prescribes maximum allowable concentration of the pollution parameters in wastewater of the solid waste landfills operated before January 01, 2010 when discharged into water sources not used for domestic water supply purpose;

- Column B2 specifies maximum allowable concentration of the pollution parameters in wastewater of the solid waste landfills newly built from January 01, 2010 when discharged into water sources not used for domestic water supply purpose.

2.2. In addition to 04 parameters specified in Table 1, depending on the requirements and pollution control purposes, the values of other pollution parameters applied in the provisions of QCVN 24: 2009/BTNMT- National Technical Regulation of industrial waste water without application of the coefficient of flow/capacity of the wastewater receiving source (Kq) and coefficient by waste source flow (Kf) to calculate the maximum value of the pollution parameters in wastewater of the solid waste landfills (Apply Cmax = C).

3. METHOD FOR DETERMINIATION

3.1. Method of determining the concentration of the pollution parameters in wastewater of the solid waste landfills specified in this Regulation shall comply with the following national standards:

- TCVN 6001-1:2008 Water quality - Determination of oxidation demand after n days (BODn) - Part 1: Methods for dilution and culture supplemented with allylthiourea;

-  TCVN 6491:1999 (ISO 6060:1989) Water quality - Determination of the chemical oxygen demand (COD);

-  TCVN 61791-1:1996 (ISO 71501:1984)  Water quality  Determination of ammonium Part 1: automated spectrometric method;

- TCVN 5988:1995 (ISO 5664-1984) Water quality - Determination of ammonium - Distillation and titration method;

- TCVN 6638:2000 Water quality - Determination of nitrogen - Inorganic catalytic method after deoxidizing by Devarda alloy;

3.2. When there are no national standards for determining the value of the pollution parameters in wastewater of solid waste landfills specified in this Regulation, apply international standards of equivalent or higher accuracy.

4. IMPLEMENTATION ORGANIZATION

4.1. This Regulation regulates separately for waste water of the solid waste landfills and replaces the application of the Vietnamese standard TCVN 5945:2005 on industrial waste water – waste standards issued together with the Decision No. 22/2006/QD-BTNMT dated December 18, 2006 of the Minister of Natural Resources and Environment for the enforcement of application of Vietnam standards on Environment.

4.2. State management agencies on environment are responsible for guidance, inspection and supervision of the implementation of this Regulation.

4.3. Where the national standards on methods of determination cited in Section 3.1 of this Regulation are amended, supplemented or replaced, then comply with the new standards.